#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <SPI.h>

#include <MFRC522.h>

#include <SoftwareSerial.h>

// Define pins for modules and peripherals

#define RST_PIN 9      // RC522 Reset pin

#define SS_PIN 10      // RC522 SDA pin

#define BUZZER_PIN 2   // Buzzer pin

#define SWITCH_PIN 6   // Switch pin for SMS

// Initialize LCD, RFID, and SIM800L

LiquidCrystal_I2C lcd(0x27, 16, 2);

MFRC522 rfid(SS_PIN, RST_PIN);

SoftwareSerial sim800l(7, 8); // RX, TX for SIM800L

// Define item structure

struct Item {

String name;

float price;

};

// RFID tags and associated items

String knownUIDs[] = {

"43:72:f4:a3", "83:8d:b2:92", "43:57:f4:19",

"73:a3:38:25", "63:3b:65:1a", "83:1f:81:1a",

"23:bc:6b:1a"

};

Item items[] = {

{"Milk", 20.5}, {"Horlicks", 150.75}, {"Brue", 27.25},

{"Boost", 180.0}, {"Glucose", 80.75}, {"Honey", 50.25},

{"Sugar", 40.5}

};

const int itemCount = sizeof(knownUIDs) / sizeof(knownUIDs[0]);

// Variables

float total = 0.0;         // Running total of cart

String lastUID = "";       // To prevent duplicate scans

String phoneNumber = "+918925373813"; // Replace with target phone number

void setup() {

// Initialize Serial and SIM800L

Serial.begin(9600);

sim800l.begin(9600);

// Initialize LCD

lcd.init();

lcd.backlight();

lcd.print("SMART TROLLEY");

delay(2000);

lcd.clear();

lcd.print("Initializing...");

delay(3000);

// Initialize RFID

SPI.begin();

rfid.PCD_Init();

// Initialize peripherals

pinMode(BUZZER_PIN, OUTPUT);

pinMode(SWITCH_PIN, INPUT_PULLUP); // Internal pull-up resistor for switch

lcd.clear();

lcd.print("Ready to Scan!");

}

void loop() {

if(total){

// Check if the switch is pressed to send SMS

if (digitalRead(SWITCH_PIN) == LOW) { // Active LOW

sendSMS();

delay(1000); // Debounce

}

}

// Check for RFID card

if (!rfid.PICC_IsNewCardPresent() || !rfid.PICC_ReadCardSerial()) {

return;

}

// Retrieve UID from the RFID tag

String uid = getUID();

if (uid == lastUID) return; // Ignore duplicate scans

lastUID = uid;

// Match UID to known items

int index = findItemIndex(uid);

if (index != -1) {

beepBuzzer();

addItem(index);

} else {

beepBuzzer();

handleUnknownItem();

}

rfid.PICC_HaltA(); // Halt RFID to allow next scan

delay(2000);

}

// Get UID as a string

String getUID() {

String uid = "";

for (byte i = 0; i < rfid.uid.size; i++) {

uid += String(rfid.uid.uidByte[i], HEX);

if (i < rfid.uid.size - 1) uid += ":";

}

uid.toLowerCase(); // Consistent comparison

return uid;

}

// Find item index based on UID

int findItemIndex(String uid) {

for (int i = 0; i < itemCount; i++) {

if (uid == knownUIDs[i]) return i;

}

return -1; // Not found

}

// Add item to cart and update total

void addItem(int index) {

total += items[index].price;

lcd.clear();

lcd.print("Added: ");

lcd.print(items[index].name);

lcd.setCursor(0, 1);

lcd.print("Total: Rs ");

lcd.print(total);

Serial.print("Added: ");

Serial.print(items[index].name);

Serial.print(" | Price: Rs ");

Serial.println(items[index].price);

}

// Handle unknown RFID tags

void handleUnknownItem() {

lcd.clear();

lcd.print("Unknown Item!");

delay(2000);

Serial.println("Unknown RFID tag scanned.");

}

// Play a beep using the buzzer

void beepBuzzer() {

digitalWrite(BUZZER_PIN, HIGH);

delay(100);

digitalWrite(BUZZER_PIN, LOW);

}

// Send SMS with total cost

void sendSMS() {

String message = "Total cost: Rs " + String(total); // SMS content

lcd.clear();

lcd.print("Sending SMS...");

//delay(1000);

sim800l.println("AT"); // Check communication

delay(500);

sim800l.println("AT+CMGF=1"); // Text mode

delay(500);

sim800l.println("AT+CMGS=\"+918925373813\"");

delay(500);

sim800l.print(message); // Message content

delay(500);

sim800l.write(26); // End of message (CTRL+Z)

delay(5000);

lcd.clear();

lcd.print("SMS Sent!");

delay(2000);

Serial.println("SMS Sent: " + message);

}